ELECTRONIC CIRCUITS AND DEVICES LAB – ELEN 312

SPRING 2026

Experiment No. 9:  Power Amplifiers

Students’ Names and ID Nos’:	1) Mohammed Yaseen Hafiz - 100064789

2) Nevan John Thomas – 100064872

3) Tarek Alhafez - 100064737

Laboratory Section: B2				                       Date of Experiment: 20/4/26

Please note that your work for this laboratory experiment will be assessed according to the marking scheme below.  Please take this into account when conducting the experiment and writing the report.

Marking Sheet

| No. | Criteria | Description | Weight % | Mark | Comments |
| --- | --- | --- | --- | --- | --- |
| 1. | Performance, Results and Analysis | Participation and Performance of the students, Documentation and analysis of the results for each task performed in the lab. | (3*2) = 6 |  |  |
| 2. | Post Lab Assessment | Answers to assignment questions and performing post-lab analysis | 2 |  |  |
| 3. | Report presentation | The overall presentation of the report includes an Abstract, Conclusion, Analysis, Layout, and Clarity of figures, Tables, and Graphs as suggested in the report format. Also the Correct use of the English language. | 2 |  |  |
|  | Total |  | 10 |  |  |

Table of Contents

Abstract	3

Introduction	3

Equipments/Components Used	3

Experimentation for Practical & Multisim	4

Task 1: Class-A Amplifier	4

Methodology:	4

Results:	5

Analysis:	6

Task 2: Class B Amplifier:	7

Methodology:	7

Results:	8

Analysis:	8

Class AB Amplifier	9

Methodology:	9

Results:	9

Analysis:	10

Post Lab Task: Class A amplifier:	11

Methodology:	11

Results:	12

Analysis	12

Conclusion	13

# Abstract

This experiment investigates the operation and performance of power amplifiers such as Class A, Class B, and Class AB configurations. Parameters such as output voltage, current, power, and efficiency were measured and analyzed while observing waveform behavior and distortion characteristics. With Class A amplifier, we observed low distortion but poor efficiency due to continuous conduction, whereas the Class B amplifier showed significantly higher efficiency at the cost of crossover distortion. The Class AB amplifier provided a compromise by reducing distortion while maintaining relatively high efficiency. Post-lab simulation of the Class A amplifier was also conducted to compare practical and simulated results, highlighting differences due to ideal conditions in simulation. Overall, the experiment establishes the trade-offs between efficiency and signal quality in different amplifier classes.

# Introduction

This experiment studies the behavior of power amplifiers, including Class A, Class B, and Class AB. The aim is to measure important parameters like voltage, current, power, and efficiency while observing distortion in the output signal. Both practical circuits and Multisim simulations are used to understand how each amplifier works and how their performance differs.

# Equipments/Components Used

Table 1. Equipment and Components utilized

| Item | Specification/Details | Purpose |
| --- | --- | --- |
| ERFI Panel | Includes programmable DC power supplies and built-in DMM module | Provides 10 V and 5 V DC voltage. |
| NI ELVIS II Prototyping Board | Integrated breadboard area with labelled power rails | Physical construction of resistor networks |
| Fixed Resistors | Different resistance values resistor like 20kOhms, 68Ohms, and 22Ohms | To construct the circuit networks |
| NPN Transistor | A standard BD139 npn, and BD140 pnp Bipolar Junction Transistor | Used to create the appropriate DC and AC Amplifier Circuits |
| Capacitor | 1 µF capacitors | Utilized as coupling capacitors to keep the AC voltages from affecting the bias voltages and currents |
| Banana-to-alligator clip leads | Red and black for positive and ground | Connect ERFI Panel outputs and Digital Multimeter to Breadboard nodes |
| PC with Multisim Installed | NI Multisim SPICE Simulator | Circuit simulation and virtual measurements |

# Experimentation for Practical & Multisim

## Task 1: Class-A Amplifier

### Methodology:

- Build the Class A amplifier circuit using the BD139 transistor.

- Set the DC supply to 10 V.

- Apply a 1 kHz sinusoidal input signal starting at a low amplitude (~10 mV).

- Gradually increase the input until maximum output is obtained without distortion.

- Observe input and output waveforms using the oscilloscope.

- Measure the output peak-to-peak voltage Vo(p-p)and determine RMS voltage.

- Measure the DC collector current ICQ.

- Calculate input power Pi, output power Po, and efficiency η.

- Record all values.

Figure 1: Class A amplifier practical implementation

Figure 2: Class A amplifier circuit schematic

The Class A power amplifier was assembled using a BD139 NPN power transistor, a 20 kΩ base-bias resistor, a 68 Ω collector resistor, and a 1 µF input coupling capacitor. The practical circuit was built on the prototyping board according to the schematic of Figure 2, and the oscilloscope was connected to observe both the input and output waveforms.

### Results:

Table 2: Measured and calculated class A amplifier circuit values

| VCC (Supply ) (V) | ICQ (dc) (mA) | Pi (dc) Calculated (mW) | Vo(p- p) Measured (V) | VCE (RMS) Measured (V) | Po (ac) Calculated (W) | % ηCalculated |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 100 | 680 | 6.4 | 3.53 | 180 | 26.47% |

Figure 3: Output waveform observed on the oscilloscope

### Analysis:

The Class A amplifier produced a clear amplified output waveform without noticeable clipping at the selected input level. The measured output voltage was 6.4 V peak-to-peak, showing that the amplifier provided significant voltage gain. The collector current was 100 mA, which caused a relatively high DC power consumption of 680 mW. The calculated output power was 180 mW, giving an efficiency of 26.47%. This efficiency is expected for a Class A amplifier because the transistor conducts for the full 360° of the input cycle, so current flows continuously even when there is no input signal. This gives good waveform quality and low distortion, however, it also causes high power loss and low efficiency.

## Task 2: Class B Amplifier:

### Methodology:

- Construct the Class B amplifier in Multisim using BD139 and BD140 transistors.

- Set the supply voltage to 5 V.

- Apply a 1 kHz sinusoidal input signal starting at 1 V peak.

- Increase the input amplitude until maximum output without major clipping is achieved.

- Observe the output waveform and identify crossover distortion.

- Measure output voltage Vo(p-p)and RMS voltage.

- Measure output current using a current probe and determine peak current.

- Calculate DC current, input power Pi, output power Po, and efficiency η.

- Record all results.

Figure 4: Implementation of the class B amplifier in multisim

### Results:

Figure 5: Output waveform of a class B amplifier in multisim

Table 3: Measured and calculated class A amplifier circuit values using multisim

| VCC (V) | I (p-p) Measured (mA) | Ip (mA) | Idc Calculated (A) | Pi (dc) Calculated (W) | Vo (rms) Measured (V) | Vo (rms) Calculated (V) | Vo (p-p) Measured (V) | Po (ac) Calculated (W) | % η Calculated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 447 | 223.5 | 0.1423 | 0.7114 | 3.70 | 3.47844 | 9.84 | 0.549979 | 77.31% |

### Analysis:

The Class B amplifier produced a much higher efficiency compared to the Class A amplifier. The measured output voltage was 9.84 V peak-to-peak, with an RMS voltage of 3.70 V. The calculated output power was approximately 0.55 W, while the DC input power was 0.7114 W. This resulted in an efficiency of 77.31%, which is close to the theoretical maximum efficiency of a Class B amplifier. However, the waveform showed crossover distortion near the zero-crossing region. This happens because each transistor only conducts for half of the input cycle, and there is a small voltage gap before the transistors turn on. Therefore, the Class B amplifier is efficient, but it has poorer signal quality compared to Class A.

## Class AB Amplifier

### Methodology:

- Build the Class AB amplifier circuit in Multisim with BD139 and BD140 transistors and biasing components.

- Set the supply voltage to 5 V.

- Apply a 1 kHz sinusoidal input signal starting at 1 V peak.

- Increase the input until the maximum undistorted output is obtained.

- Observe input and output waveforms on the oscilloscope.

- Measure output peak, peak-to-peak, and RMS voltage.

- Measure output current using a current probe.

- Calculate DC current, input power Pi, output power Po, and efficiency η.

Figure 6: Implementation of the class AB amplifier in multisim

Record all measured and calculated values.

### Results:

Table 4: Measured and calculated class AB amplifier circuit values using multisim

| VCC (V) | Ip (mA) | Idc Calculated (mA) | Pi (dc) Calculated (W) | V0 (rms) (V) | Vo (rms) Calculated (V) | V0 (p) (V) | Po (ac) Calculated (W) | % ηCalculated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 217.5 | 0.1385 | 0.6923 | 3.51 | 3.383 | 9.57 | 0.5202 | 75% |

Figure 7: Output waveform of a class AB amplifier in multisim

### Analysis:

The Class AB amplifier gave a balanced performance between the Class A and Class B amplifiers. The measured output voltage was 9.57 V peak-to-peak, and the RMS output voltage was 3.51 V. The calculated output power was 0.5202 W, while the DC input power was 0.6923 W, giving an efficiency of about 75%. This is slightly lower than the Class B amplifier but much higher than the Class A amplifier. The main advantage of the Class AB circuit is that the biasing allows both transistors to conduct slightly around the zero-crossing point, which reduces crossover distortion. This means, Class AB provides better waveform quality than Class B while still maintaining high efficiency.

# Post Lab Task: Class A amplifier:

Figure 8: Class A amplifier circuit diagram in multisim

Methodology:

- Build the Class A amplifier circuit in Multisim using the BD139 transistor.

- Set the DC supply voltage to 10 V.

- Apply a 1 kHz sinusoidal input signal.

- Increase the input amplitude until maximum output without distortion is achieved.

- Observe waveforms using the oscilloscope.

- Measure output peak-to-peak voltage and calculate RMS voltage.

- Measure collector current ICQ.

- Calculate input power Pi, output power Po, and efficiency η.

- Record results.

### Results:

Table 5: Measured and calculated class A amplifier circuit values in multisim

| VCC (Supply ) (V) | ICQ (dc) (V) | Pi (dc) Calculated (W) | Vo(p-p) Measured (V) | VCE (RMS) Measured (V) | Po (ac) Calculated (mW) | % ηCalculated |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 56.5 | 0.565 | 7.91 | 2.7966 | 353 | 62.47% |

Figure 9: Output results for class A amplifier from within multism

### Analysis

The Multisim Class A amplifier produced a larger output voltage than the practical circuit, with a measured value of 7.91 V peak-to-peak. The collector current was 56.5 mA, giving a DC input power of 0.565 W. The calculated AC output power was 353 mW, and the efficiency was 62.47%. This efficiency is much higher than the practical Class A result, which may be due to ideal simulation conditions, ideal component behavior, and differences between the simulated and real transistor parameters. In real circuits, losses occur due to transistor heating, component tolerances, wiring resistance, and measurement limitations. The simulated waveform was clean and undistorted, showing that the Class A amplifier operated properly, but the efficiency result is higher than normally expected for a practical Class A amplifier.

# Conclusion

The results show that each amplifier type has its own advantages: Class A has good signal quality but low efficiency, Class B is more efficient but introduces distortion, and Class AB provides a balance between both. The practical and simulation results are fairly close, with small differences due to real components. Overall, the experiment helps in understanding which amplifier is suitable for different applications.